InnoCart

Comprehensive Research & Technical Document

Phase 1 (NFC-Based Smart Cart)  →  Phase 2 (InnoBasket V2 – UHF RFID)

SWIPE Technologies  |  March 2025 – Present

| Field | Details |
| --- | --- |
| Project Name | InnoCart (Innocart) |
| Organization | SWIPE Technologies |
| Phase 1 Period | March 2025 – January 2026 |
| Phase 2 Period | January 2026 – Ongoing |
| Domain | Smart Retail / IoT / AI-Powered Shopping |
| Technology | NFC, UHF RFID, ESP32, BLE, Flutter, Node.js, MongoDB |
| Target Markets | Supermarkets, Apparel & Fashion Retail, Airport Shopping |

# 1. Executive Summary

InnoCart is a smart autonomous checkout and retail intelligence platform developed by SWIPE Technologies. It eliminates checkout queues and transforms ordinary shopping carts into data-gathering business intelligence tools. The project has evolved through multiple phases — from a prototype NFC-based grocery cart to a sophisticated UHF RFID-powered system with AI-driven fashion recommendations and real-time behavioural analytics.

The platform addresses two critical pain points in physical retail: (1) operational inefficiencies caused by checkout queues, and (2) the 'data blindspot' — whereby physical retailers lack the granular shopper behaviour insights that e-commerce competitors possess for every browsed product and keyword search.

InnoBasket V2 bridges the gap between the physical and digital retail worlds, offering a checkout-free experience for shoppers and a data-rich ecosystem for retailers.

# 2. Project Versions & Evolution

## 2.1  InnoCart V1 — Smart Grocery Cart  (Mar 2025 – Jan 2026)

The first version focused on building a smart shopping cart for grocery stores. It used NFC (Near-Field Communication) tags attached to products and a cart-mounted scanner, allowing customers to drop items into the cart and have them automatically tracked and billed through a companion mobile application. The cart communicated with the customer's phone via Bluetooth Low Energy (BLE), sending product information in real time and enabling UPI-based checkout without visiting a cashier.

## 2.2  InnoCart V2 — InnoBasket V2  (Jan 2026 – Present)

Version 2 is a B2B-oriented proof-of-concept (POC) that dramatically upgrades the hardware and intelligence of the system. The key change is the migration from HF NFC (13.56 MHz) to UHF RFID (865–867 MHz, Indian band), enabling multi-tag simultaneous scanning at up to 900 tags/sec. V2 introduces:

- A Faraday-shielded resonant cavity chassis that traps and focuses RF energy for zero cross-reads from neighbouring carts

- Dual-processor architecture (Arduino MEGA 2560 + ESP32-S3)

- AI-driven product recommendations and fashion pairing (targeting Zudio, Trends, Pantaloons)

- Real-time behavioural analytics, lost-sale tracking, and heat maps for retailers

- Seamless UPI payment integration with on-cart TFT display

## 2.3  InnoCart V3 — AI Fashion & Fit Advisor  (Planned)

Version 3 is envisioned as a fashion-centric evolution, incorporating facial feature analysis and fit-related AI suggestions to improve sales for fashion retail chains like Zudio.

# 3. Hardware Architecture

## 3.1  Phase 1 – NFC-Based Prototype

The Phase 1 prototype was built around the following hardware stack:

| Component | Part Number | Role |
| --- | --- | --- |
| Microcontroller (Main) | ESP32-S3-WROOM-1-N16R8 | Main control, Bluetooth, peripheral interfacing |
| BLE Module | Built into ESP32-S3 | Cart-to-mobile app communication |
| NFC Reader | PN532 (I2C/SPI) | Scanning NFC-tagged products and security NFC card |
| NFC Reader (V2) | PN5180 / ISO15693 | Upgraded HF reader, SPI-only, up to 7 Mbit/s |
| Servo Motor | MG995 | Flap mechanism for product placement |
| NFC Tags | NTAG213 / NTAG210 | Attached to products; ~64 bytes of memory |
| Battery Pack | 3S, 12V nominal | Primary power supply |
| BMS | 2A BMS | Protects and manages lithium cells |
| Buck Converters | 12V → 5V / 3.3V | Steps down to module power consumption |
| Enclosure | Solid 3D-printed case | Plug-and-play housing for the cart |

PN5180 Interface Details

| Feature | Description |
| --- | --- |
| Interface | SPI only (no I2C or UART support) |
| Speed | Up to 7 Mbit/s |
| Voltage | 3.3V |
| Supported Protocols | ISO 14443 (A/B), ISO 15693, ISO 18000-3 |

PN5180 Pin Mapping

| Pin | Function |
| --- | --- |
| SCK | Clock |
| MISO | Master In |
| MOSI | Master Out |
| CS (NSS) | Chip Select |
| BUSY | Reader Status |
| RESET | Reset |
| IRQ | Interrupt |
| VCC | Power |
| GND | Ground |

## 3.2  Phase 2 – UHF RFID Architecture (InnoBasket V2)

Phase 2 dramatically overhauls the scanner hardware to UHF RFID, enabling simultaneous multi-tag detection suitable for high-density fashion retail environments.

Dual-Processor Architecture

- RFID Core (Arduino MEGA 2560): Dedicated to running the IMPINJ E710 4-port UHF reader, handling complex RF protocols, antenna switching, multiplexing, and ensuring industrial-grade read accuracy.

- Intelligence Core (ESP32-S3-WROOM-1-N16R8): Receives clean EPC (Electronic Product Code) IDs via a UART bridge. Executes edge AI algorithms for product validation, Wi-Fi connectivity, and cloud synchronisation.

UHF RFID Reader – IMPINJ FM-E710 (Fonkan / EasyRFID E21316M-4)

| Specification | Value |
| --- | --- |
| Chipset | IMPINJ E710 |
| Protocol | ISO 18000-6C (EPC Gen 2 v1 and v2) |
| Frequency (India) | 865 – 867 MHz |
| Peak Inventory Rate | 900 tags/sec |
| Max Reading Distance | 35m (based on antenna) |
| Interface | TTL UART (Tx, Rx) |
| Communication Port | 6-pin FPC → TTL UART |
| Baud Rate | 115200 bps |
| Antenna Ports | 4 ports, 50Ω RF, MMCX Female |
| Max Current | 1600 mA (peak), ≤300 mA average |
| Standby Current | ≤1 mA |
| Temperature Protection | Works <85°C; auto-stops above, resumes when cooled |
| Estimated Cost | ₹6,122 (module) + ₹1,775 (shipping) = ₹7,897 |

Antenna Selection (Purchased from Alibaba – Shenzhen Fonkan)

Antennas operate on the 865–868 MHz Indian UHF band with SMA male connectors.

| Model | Gain | Size | Unit Cost (₹) | Qty |
| --- | --- | --- | --- | --- |
| FA-303 | 3 dBi | 5cm × 8cm | 649 | 3 |
| FA-304 | 4 dBi | 7cm × 7cm | 881 | 1 |
| FA-305 | 5 dBi | 9cm × 9cm | 1066 | 1 |

Total estimated antenna cost: ₹3,894 (product) + ₹3,238 (shipping) = ₹7,134

# 4. Product Tagging & NFC/RFID Technology

## 4.1  Tag Evolution

Phase 1 used HF NFC tags (NTAG213 / NTAG210) operating at 13.56 MHz with protocols ISO 14443-3A and ISO 14443-0. These have approximately 64 bytes of user memory and are suitable for individual item scanning.

Phase 2 transitions to UHF RFID EPC tags (ISO 18000-6C, Class 1 Gen 2) operating at 865–867 MHz (Indian UHF band). The switch to UHF enables simultaneous detection of multiple items dropped into the cart, dramatically improving speed and enabling bulk scanning suitable for fashion retail.

## 4.2  RF Frequency Classification

| Band | Frequency Range | Used In |
| --- | --- | --- |
| LF | 30 kHz – 300 kHz | Animal tracking, access control |
| HF | 3 MHz – 30 MHz (13.56 MHz) | NFC, ISO 14443, Phase 1 InnoCart |
| UHF | 300 MHz – 3 GHz (865–967 MHz) | EPC Gen2, Phase 2 InnoCart, retail RFID |
| UHF (India) | 865 – 867 MHz | InnoBasket V2 target band |

## 4.3  Reader Research History

The team evaluated multiple readers before settling on the current solution:

- MFRC522 – HF RFID, 13.56 MHz; basic entry-level reader

- PN532 – NFC, 13.56 MHz; used in Phase 1 prototype

- PN5183 – HF RFID, ISO 15693; considered as upgrade to PN532

- PN5180 – HF RFID; SPI-only; supports 10 tags simultaneously; no I2C/UART (limitation)

- R200 – UHF RFID, 863–965 MHz; single-antenna input only; limited to 0–1m range with 3dBi antenna

- RD161 / IMPING E710 – Completed product; no IP/copyright issues; selected as final reader

- IMPING E710 (FM-704, Fonkan) – Final chosen module; 4-port, 900 tags/sec

## 4.4  Dual Antenna Concept

Using a single PN5180 chip, two antennas can be operated simultaneously. This is particularly useful when a product is incorrectly dropped into the cart, allowing detection from multiple angles. The concept leverages the SPI serial bus for antenna multiplexing.

# 5. Shielded Pod – Faraday Resonant Cavity

One of the key innovations in InnoBasket V2 is the Faraday-shielded chassis. Conventional smart carts suffer from signal leakage and cross-talk in dense store aisles, causing false reads from neighbouring carts.

## 5.1  Concept

The cart bin is lined entirely with adhesive aluminium tape / aluminium foil to create a resonant cavity (Faraday cage effect). RF energy from the UHF reader is trapped inside the bin, creating a 'microwave effect' that forces waves to bounce and hit all tags from every angle.

## 5.2  Benefits

- Maximum Read Accuracy: Detects hidden tags via reflected waves

- Zero Cross-Reads: Prevents scanning products outside the cart

- Lower Power: Operates efficiently at 18 dBm instead of maximum power

## 5.3  Construction

- Apply adhesive aluminium tape/foil to cover the bin entirely — no holes, no leakage

- Even a single hole causes RF disturbances; UHF waves must bounce inside the bin

- Safety: Since aluminium is conductive, the ESP32 and reader module must be covered with plastic casing to avoid short circuits

# 6. Software Stack

## 6.1  Phase 2 (Current) Software Stack

| Layer | Technology |
| --- | --- |
| Firmware / RFID Core | Arduino IDE (C/C++) on Arduino MEGA 2560 — drives IMPINJ E710 UHF reader |
| Intelligence Core | ESP32-S3-WROOM firmware — handles serial comms, Wi-Fi, display, WebSockets |
| On-Cart Display | 7-inch touch display driven by ESP32-S3 — live bill, AI styling, payment QR, rush deals |
| Cart-Server Communication | HTTP requests (product fetch, styling API) + WebSockets (payment callback, 2-min window) |
| Mobile App (Flutter) | Session initiation via QR scan; receives pushed PDF bill post-payment |
| Backend | Node.js + MongoDB |
| Payment Gateway | UPI QR (Razorpay) — rendered on cart display; supports GPay, PhonePe, Paytm |
| AI Style Engine | Server-side AI API — processes EPC ID + customer profile → top 3 styling suggestions |
| Bill Generation | Server-generated PDF pushed to mobile app after payment confirmation |

## 6.2  Arduino IDE Libraries Used

| Library | Function | Author |
| --- | --- | --- |
| NfcPN5180.h | PN5180 NFC reader | Playful Technologies |
| NfcPN532.h | PN532 NFC reader | Adafruit |
| NfcPN532.h (alt) | PN532 NFC (additional) | Kevin Harrington, Bennett |
| ESP32Servo.h | Servo control | Rob Tillaart |
| HX711.h | Weight sensor | Frank de Brabander |
| LiquidCrystal_I2C.h | LCD display | Various |

# 7. Display & Mobile Application Architecture

## 7.1  Architecture Shift: BLE Removed — On-Cart 7-inch Touch Display

In the current phase (InnoBasket V2), the live bill display has been moved entirely off the customer's mobile and onto a dedicated 7-inch touch display mounted on the cart itself. This eliminates BLE pairing friction and makes the shopping experience fully self-contained on the cart hardware.

| Capability | Phase 1 (BLE + Mobile) | Phase 2 (On-Cart Display) |
| --- | --- | --- |
| Live bill display | Customer mobile phone via BLE | 7-inch touch display on cart |
| Cart pairing | BLE auto-connect to nearest cart | QR code scan at session start |
| Product scan feedback | Mobile app notification | On-cart display instant update |
| AI styling | Not available | Go with AI Styling button on cart screen |
| Payment QR | Razorpay in mobile app | UPI QR rendered on cart display |
| Bill delivery | PDF downloaded on mobile | Pushed to the mobile app linked at session start |
| Rush deals / coupons | In-app push | Displayed on cart screen at checkout |

## 7.2  Mobile Application Role in Current Architecture

The mobile app is now a lightweight session-binding and post-purchase tool rather than a real-time cart interface. Its responsibilities are:

- QR Code Scanning: Customer scans the cart QR at session start to link their account to the cart session on the server

- Bill Receipt: After payment, the server pushes the itemised PDF bill directly to the customer's linked mobile account

- Order History: Customer can review all past purchases, filterable by date

- Profile & Preferences: Name, email, mobile; AI style profile (skin tone, fit preferences) stored for personalisation

- NFC Security Check: App shows verification prompt at exit — customer taps NFC security tag to confirm purchased items

- Notification Alerts: Payment success/failure and post-purchase confirmation

- Navigation in Store (Phase 2): RSSI-based indoor navigation without staff

- Voice Assistant (Phase 2): For hearing-impaired customers; English, Telugu, Kannada, Hindi

# 8. Database Schema

The InnoCart backend uses MongoDB with the following entity model:

## 8.1  Customers

| Field | Type | Notes |
| --- | --- | --- |
| customer_id | PK | Primary key |
| name | String |  |
| email | String |  |
| mobile | String |  |
| password_hashed | String | Hashed |
| created_at | DateTime |  |

## 8.2  Orders

| Field | Type | Notes |
| --- | --- | --- |
| order_id | PK | Primary key |
| customer_id | FK | References Customers |
| cart_id | String | Physical cart identifier |
| transaction_id | String | Razorpay transaction ref |
| payment_status | Enum | pending / success / failed |
| total_amount | Float | Including taxes |
| date_time | DateTime | Timestamp of order |

## 8.3  Products

| Field | Type | Notes |
| --- | --- | --- |
| product_id | PK | Primary key |
| category | String |  |
| name | String |  |
| price | Float |  |
| nfc_tag_id | String | Linked NFC/RFID tag ID |

## 8.4  Additional Entities

- Order_Items: order_item_id (PK), order_id (FK), product_id (FK), quantity, price_at_purchase, subtotal

- Inventory: inventory_id (PK), product_id (FK), stock, last_updated

- Admins: store_id (FK), admin_id (PK), name, email, pass_hash, role, created_at, last_login

- Stores: store_id (PK), name, location, contact

# 9. Web Application Architecture

## 9.1  Customer-Facing Pages (PHP)

| Page/File | Purpose |
| --- | --- |
| index.php | Landing page – About InnoCart, animations, links to register/login |
| login.php | Customer login (mobile/email + password) |
| register.php | Customer registration (name, email, phone, password) |
| logout.php | Customer logout session |
| cart.php | Live cart page – BLE connect, dynamic bill, add/remove items |
| bill.php | Bill summary with order items, total |
| payment.php | UPI / Razorpay payment processing |
| payment_success.php | Verifies payment status after callback |
| order_history.php | View past orders and order items |
| profile.php | Customer profile – view/update details |

## 9.2  Admin Interface

- Admin Login: Secure login with email + password

- Dashboard Overview: KPIs – sales, active carts, orders; quick access to inventory alerts

- Product Management: CRUD operations on products; assign products to stores

- Inventory Management: Adjust stock quantities, view inventory logs, track stockin/stockout

- Orders Management: List all orders (filterable by status/date); expand into order items

- Customer Management: View customers; see order history per customer

- Reports / Analytics: Sales trends, popular products, low stock alerts

# 10. Customer Workflow (End-to-End)

The complete end-to-end shopping journey on InnoBasket V2 — fully driven by the on-cart 7-inch touch display:

Phase A — Session Initiation

- Step 1: Customer enters the store and picks up an InnoBasket cart

- Step 2: Customer opens the InnoCart mobile app and scans the QR code displayed on the cart screen

- Step 3: The server links the customer's account to that specific cart session — the cart now knows who is shopping

- Step 4: The 7-inch on-cart display activates into shopping mode, showing a live empty bill (₹0)

Phase B — Shopping & Product Detection

- Step 5: Customer picks garments and drops them into the Faraday-shielded cart bin

- Step 6: The UHF RFID reader (IMPINJ E710, on dedicated Arduino MEGA board) instantly reads EPC tag IDs from dropped items

- Step 7: Arduino sends EPC IDs to ESP32-S3 via UART serial communication

- Step 8: ESP32-S3 sends an HTTP request to the server with the EPC ID to fetch product details (name, price, category, image)

- Step 9: Server responds with product data; ESP32-S3 updates the live bill on the on-cart display in real time — item is added with name, price and running total

- Step 10: Customer can remove items at any time via the on-cart display; removed items are tracked server-side for lost-sale analytics

Phase C — AI Styling Engine (Optional)

- Step 11: At any point during shopping, customer taps the 'Go with AI Styling' button on the cart's touch display

- Step 12: The display shows the list of items currently in the cart and asks: 'Which item do you want styling for?'

- Step 13: Customer selects one item (e.g., a blue kurta)

- Step 14: Cart sends a styling request to the server with the selected item's EPC ID and the customer's style profile

- Step 15: Server's AI engine analyses the item and the customer's profile, then returns the top 3 outfit pairings with aisle navigation hints

- Step 16: Customer sees the 3 AI style suggestions on the cart display and can navigate to those aisles to pick matching items

Phase D — Checkout: Rush Deals, Coupons & Upselling

- Step 17: When the customer is done shopping, the cart display transitions to the checkout screen

- Step 18: The server pushes personalised Rush Deals and available coupons for this customer based on their cart contents and purchase history

- Step 19: Customer reviews and applies coupons — the total amount updates on the display (upselling / discount realisation happens here)

Phase E — Payment

- Step 20: Cart initialises a WebSocket connection (2-minute window) and requests a UPI payment QR from the server for the final total amount

- Step 21: UPI QR code is rendered on the 7-inch cart display — customer scans it using any UPI app (GPay, PhonePe, Paytm, etc.)

- Step 22: On payment success, the WebSocket receives the confirmation — the cart display plays a success animation and payment_status is updated to 'success' on the server

- Step 23: On payment failure or timeout (>2 minutes), the display shows a failure animation and the customer can retry

Phase F — Bill Delivery & Security Exit

- Step 24: Server automatically pushes the itemised PDF bill to the customer's mobile app (linked at session start via QR scan)

- Step 25: Order history is saved to the customer's account

- Step 26: Customer walks to the store exit; the mobile app prompts the NFC security check

- Step 27: Customer taps the NFC security tag at the exit — the system verifies the purchased items against the session bill

- Step 28: Security confirms and the customer exits seamlessly — no manual billing, no queue

# 11. InnoBasket V2 – Advanced Technical Details

InnoBasket V2 is positioned as a B2B SaaS product. Its full title: "InnoBasket V2: An AI-Powered, Shielded Autonomous Checkout Ecosystem with Real-Time Behavioural Analytics (Product Based)"

## 11.1  Serial Handoff & On-Cart Communication Flow

The data pipeline from physical item to on-cart display works as follows:

- UHF RFID tags (EPC Gen 2, ISO 18000-6C) on garments are detected by the IMPINJ E710 4-port UHF reader, which is connected to and managed by the Arduino MEGA 2560 (RFID Core)

- The Arduino MEGA sends the raw EPC ID to the ESP32-S3 (Intelligence Core) via UART serial communication (Tx/Rx bridge)

- The ESP32-S3 fires an HTTP request to the server over Wi-Fi, passing the EPC ID to fetch product details (name, price, category, image)

- The server queries the Products collection and returns the product data to ESP32-S3

- ESP32-S3 updates the 7-inch on-cart touch display: the new item appears in the live bill with its name, price, and updated running total

- As items are dropped continuously, each triggers the same pipeline independently, keeping the display in real time

- When items are removed via the display, ESP32-S3 notifies the server — removal is logged for lost-sale analytics

After shopping is complete:

- The cart screen transitions to checkout — server pushes Rush Deals and coupons to the cart display via the active HTTP/WebSocket connection

- After coupon application, ESP32-S3 requests a UPI payment QR for the final total from the server

- The server generates a Razorpay/UPI QR and returns it; the QR is rendered on the cart display

- A WebSocket connection is opened (2-minute window) to listen for the payment callback

- On successful UPI payment, the WebSocket receives confirmation — cart display plays success animation; payment_status updated to 'success' in the Orders collection

- On timeout or failure, a failure animation is shown and the customer can retry

- Server pushes the PDF bill to the mobile app linked at session start

## 11.2  AI Style Engine — Detailed Flow

The AI Style Engine is triggered on-demand from the cart's touch display and operates as a client-server interaction:

- Customer presses 'Go with AI Styling' on the 7-inch cart display during shopping

- Cart display renders the current cart item list and prompts: 'Which item do you need styling for?'

- Customer selects one item (tap to choose)

- ESP32-S3 sends a styling API request to the server with: (a) the selected item's EPC ID, (b) the customer's style profile from their account (skin tone, fit preferences, purchase history)

- Server AI engine processes the request — analyses the selected item's category, colour, and style attributes against the customer profile and inventory data

- Server returns top 3 AI styling suggestions, each with: matching item name, aisle location in store, and a brief style note

- Cart display renders the 3 suggestions in a visually clear card layout

- Customer can navigate to suggested aisles and add those items to the cart, continuing the cycle

This entire loop runs on the cart display — no mobile phone interaction required. The AI style engine also feeds the recommendation data back to the retailer's analytics dashboard, enabling insights on which styling suggestions drive additional purchases.

## 11.3  Lost Sale Analytics (Retailer POV)

By tracking the state of every item (added or removed before checkout), the system generates 'Lost Sale Analytics'. This provides retailers with granular data on why products are rejected (e.g., price sensitivity vs. fit issues) before checkout. This transforms the shopping cart from a passive carrier into an active business intelligence tool, allowing offline retailers to optimise inventory and pricing with the precision of an online algorithm.

## 11.4  Indoor Positioning / Navigation System

RSSI Fingerprinting is implemented on ESP32-S3 using Wi-Fi and BLE. Multiple signal strengths from multiple access points are received, and the position is estimated via machine learning (trilateration). This provides the indoor navigation system for the store. The MQTT protocol (Message Queuing Telemetry Transport) is used as the lightweight publish-subscribe messaging protocol essential for IoT.

GitHub reference: avebn/indoor-positioning-trilateration

# 12. Market Analysis & Target Domains

## 12.1  Supermarkets

- Adoption rate is currently low; tag costs are high (near ₹10/chip)

- Reliance already uses UHF RFID for store and stock management, but tags are only kept for expensive items (≥₹100)

- UHF RFID tagging trends are increasing in this domain

- Future potential: tag costs may become negligible as adoption scales

## 12.2  Apparel & Fashion Retail (Primary Target)

- Tag cost is negligible — many stores (Zudio, Pantaloons, Decathlon, Style Union) already use UHF RFID tags

- This domain is very profitable once the cart design is finalised

- At festive seasons and peak discount sales, counters get very busy — InnoCart eliminates queues

- Pyramid pop-up offers delivered directly on the mobile app increase sales of discounted items

- InnoCart V3 trial version planned for Zudio

## 12.3  Airport Shopping

- Travellers in a hurry need a companion to select garments for them quickly

- Fast fashion retail companies (Zudio, Trends, Pantaloons) are natural airport partners

## 12.4  InnoCart V3 – Unique Fashion Features

- AI Fashion Recommendations: If a person takes a blue kurta, the system gives an AI suggestion: 'try white chinos in aisle 4; suits your kurta once you try that'

- Customer account stores skin tone, facial features, and fit data

- Based on that data and cart-dropped items, AI analyses and gives suggestions

- Rush Deals: Time-limited real-time discount offers based on cart value

# 13. Financial Projections (3-Year Forecasting)

Partnership target: Zudio, with 150+ stores. Each store estimated to need approximately 30 carts.

| Item | Amount |
| --- | --- |
| Stores | 150 |
| Carts per store | 30 |
| Total carts | 4,500 |
| Estimated cart value | ₹15,000 per cart |
| Cart cost (hardware) | ₹4,50,000 (carts) + ₹2,00,000 (server/routers) |
| SaaS & dashboard charge | ₹30,000/month per store |
| Per store 3-year revenue | ₹10,80,000 |
| Total 3-year (150 stores) | ₹25,80,00,000 (~₹26 Crores) |

# 14. Team Structure

| Member | Role |
| --- | --- |
| Mani Tejas | Hardware (Embedded Systems / Firmware) |
| Sai Sahithi U. | Application Development (Frontend) |
| T.E. Jagan + Mani Kavita | Backend (Database, Logic, Architecture) |
| Shashank + Vijay Bhaskar | UI/UX – Mobile App, Logo Designing, Cart Design, Presentation |
| Madhu (Mathu) | Embedded Systems |

## Research References

- Market Analysis – understanding supermarket/fashion retail landscape

- Competitive Analysis – Amazon Go, Walmart self-checkout research

- Existing smart-cart systems research

- Feedough AI – market research / customer acquisition / tracking reach

# 15. Technical Keywords & Glossary

| Term | Definition |
| --- | --- |
| UHF RFID | Ultra-High Frequency Radio Frequency Identification; 865–967 MHz; EPC Gen 2 |
| EPC Gen 2 | Electronic Product Code; ISO 18000-6C standard for UHF RFID |
| Faraday Cage | Conductive enclosure that blocks external electromagnetic fields |
| Resonant Cavity | Structure that traps and amplifies RF energy via internal wave reflection |
| BLE | Bluetooth Low Energy; used for cart-to-mobile communication |
| RSSI Fingerprinting | Indoor positioning via received signal strength from multiple BLE/Wi-Fi access points |
| MQTT | Message Queuing Telemetry Transport; lightweight IoT messaging protocol |
| SPI | Serial Peripheral Interface; communication protocol used by PN5180 |
| UART | Universal Asynchronous Receiver/Transmitter; serial comms between Arduino and ESP32 |
| Lost Sale Analytics | Tracking items added then removed from cart before checkout to identify rejection reasons |
| Upselling | AI-driven suggestion of complementary products to increase average ticket value |
| RG316 Coaxial | High integrity coaxial cable used between reader and antennas |

# 16. Conclusion

InnoCart / InnoBasket V2 represents a technically sophisticated, market-validated approach to solving physical retail's two biggest problems: checkout friction and data blindness. Starting from a university-level NFC prototype, the project has evolved into a B2B-grade UHF RFID system with Faraday-shielded hardware, dual-processor architecture, AI recommendation engines, and real-time behavioural analytics.

The platform's potential is particularly strong in the Indian fast-fashion retail sector (Zudio, Pantaloons, Trends), where UHF RFID tags are already in use, the tag cost is negligible, and checkout counters face peak-hour congestion. The 3-year financial projection of approximately ₹26 Crores across 150 stores, delivered as a SaaS model with hardware leasing, demonstrates a clear path to commercial viability.

InnoBasket V2 bridges the gap between the physical and digital worlds — offering a checkout-free experience for users and a data-rich, AI-powered intelligence ecosystem for the business.

— End of Document —