InnoCart V2 — Circuit Design Document

SWIPE Technologies  ·  UHF RFID Autonomous Checkout  ·  Phase 3

Generated: 08-May-2026  ·  Hardware Simulation Phase Complete

# 1. System Communication Map

The diagram below represents the top-level communication architecture of InnoCart V2. All blocks and signal paths match the hand-drawn circuit schematic (9/5/26).

E710 ──UART──► Arduino Mega 2560 ──UART──► ESP32-S3 ──I2C──► 7" Display Antennas (×4) ──SMA──► E710 (4 RF ports) PIR / IR ──GPIO──► ESP32-S3 ──GPIO──► Arduino EN pin ──► E710 EN 3S2P Battery ──► BMS ──► Buck Converters (×3) ──► 5V Rails + LDO ──► 3.3V Rail Wi-Fi (ESP32-S3) ──► SWIPE Main Store Server (HTTP / WebSocket)

# 2. Power System

## 2.1  Battery Pack

Configuration: 3S2P Li-Ion (6× Samsung 32E / LG MJ1 18650 cells).  Nominal voltage: 11.1V.  Capacity: 5200mAh.

## 2.2  BMS — 3S 10A Battery Management Module

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| Over-voltage protection | 4.25V / cell | Under-voltage protection | 2.75V / cell |
| Over-current cutoff | 10A | Short circuit protect | Yes (instant cutoff) |

## 2.3  Power Rails Summary

| Rail | Voltage | Source | Max I | Loads |
| --- | --- | --- | --- | --- |
| 11.1V BUS | 11.1V | BT1 + BMS1 | 10A | All buck converters + charger input |
| 5V_E710 | 5.0V | Buck_A (U1) | 2A | Impinj E710 module ONLY (isolated) |
| 5V_LOGIC | 5.0V | Buck_B (U2) | 1A | Arduino Mega + IR sensors + Level Shifter HV |
| 5V_DISPLAY | 5.0V | Buck_C (U3) | 0.5A | 7" display ONLY (isolated) |
| 3.3V_RAIL | 3.3V | AMS1117 (U4) | 0.8A | ESP32-S3 + PIR sensor + LS LV + MAX17043 |

⚠  NEVER share the 5V_E710 rail with any other load. The E710 PA draws up to 1600mA in burst — this rail is isolated by design.

## 2.4  Charging Circuit — TP5100

Input: 12V DC via 5.5mm barrel jack (J1). Charge current set by R_PROG = 600Ω → Icharge = 2A. Status LEDs: D1 (charging, red) + D2 (standby, green) with 1kΩ series resistors.

## 2.5  Fuel Gauge — MAX17043 (I2C)

Address: 0x36. Connected to ESP32-S3 I2C bus (SDA: GPIO8, SCL: GPIO9). Low-battery alert interrupt on ESP32 GPIO4.

# 3. Impinj E710 UHF RFID Module (U7)

Module: EASYRFID FM-E710. Interface: 12-pin FPC connector, UART 3.3V TTL at 115200 baud. Four MMCX antenna ports via pigtail adapters to SMA.

## 3.1  E710 Pin Mapping

| Pin | Signal | Dir | Connected To | Voltage |
| --- | --- | --- | --- | --- |
| 1, 2 | VCC (5V) | IN | 5V_E710 (Buck_A) — dual supply pins | 5V |
| 3, 4 | GND | — | Common GND | 0V |
| 5 | EN (Enable) | IN | Level Shifter U8 LV_A3 ← Mega D22 | 3.3V MAX |
| 6 | GPIO1 | OUT | Level Shifter U8 LV_A4 → Mega D23 | 3.3V |
| 7, 8 | GPIO2, GPIO3 | — | NC (not connected) | — |
| 9 | UART_RX | IN | Level Shifter U8 LV_A1 ← Mega TX1 (shifted) | 3.3V |
| 10 | UART_TX | OUT | Level Shifter U8 LV_A2 → Mega RX1 | 3.3V |
| 11, 12 | GPIO4, GPIO5 | — | NC (not connected) | — |
| ANT1–4 | RF Ports | RF | MMCX pigtails → RG316 coax → Antennas | 50Ω RF |

## 3.2  Antenna Configuration

| Port | Antenna | Model | Gain | Position |
| --- | --- | --- | --- | --- |
| ANT PORT 1 | ANT1 | YANPODO YPD80907 | 5dBi | Left wall of basket |
| ANT PORT 2 | ANT2 | YANPODO YPD80907 | 5dBi | Right wall of basket |
| ANT PORT 3 | ANT3 | A001 Circular | 8dBi | Floor — facing up |
| ANT PORT 4 | ANT4 | A001 Circular | 8dBi | Display rear |

RF cable: RG316 50Ω coax, 35cm per port. Connector: MMCX male → SMA female pigtail × 4.

# 4. Logic Level Shifter (U8)

IC: BSS138-based 4-channel bidirectional (Sparkfun BOB-12009 or equivalent). HV side: 5V_LOGIC. LV side: 3.3V_RAIL.

⚠  CRITICAL — E710 UART is 3.3V TTL. Arduino Mega is 5V logic. Without this shifter, 5V from Mega TX1 will damage E710 UART_RX permanently.

| Channel | HV Side (5V) | LV Side (3.3V) | Purpose |
| --- | --- | --- | --- |
| CH1 | Mega TX1 (PIN18) → HV_B1 | LV_A1 → E710 PIN9 UART_RX | UART TX: Mega commands to E710 |
| CH2 | HV_B2 → Mega RX1 (PIN19) | E710 PIN10 UART_TX → LV_A2 | UART RX: E710 EPC data to Mega |
| CH3 | Mega D22 → HV_B3 | LV_A3 → E710 PIN5 EN | Enable/disable E710 RF PA |
| CH4 | HV_B4 → Mega D23 | E710 PIN6 GPIO1 → LV_A4 | Antenna presence monitor (spare) |

# 5. Arduino Mega 2560 (U9) — RFID Slave Processor

Role: UART bridge between E710 and ESP32-S3. Handles E710 enable control, EPC frame parsing, and relay to main controller. Powered from 5V_LOGIC rail via VIN pin.

## 5.1  Arduino Mega Pin Mapping

| Pin | Function | Dir | Connected To | Voltage |
| --- | --- | --- | --- | --- |
| VIN | Power IN | OUT→ | 5V_LOGIC (Buck_B) | 5V |
| GND | Ground | — | Common GND | 0V |
| TX1 (PIN 18) | Serial1 TX | OUT→ | Level Shifter U8 HV_B1 → E710 UART_RX | 5V→3.3V |
| RX1 (PIN 19) | Serial1 RX | ←IN | Level Shifter U8 HV_B2 ← E710 UART_TX | 5V |
| TX2 (PIN 16) | Serial2 TX | OUT→ | R6/R7 divider → ESP32-S3 GPIO44 | 5V→3.3V |
| RX2 (PIN 17) | Serial2 RX | ←IN | ESP32-S3 GPIO43 TX | 3.3V |
| D22 | E710 EN ctrl | OUT→ | Level Shifter U8 HV_B3 → E710 EN | 5V→3.3V |
| D23 | E710 GPIO1 mon | ←IN | Level Shifter U8 HV_B4 ← E710 GPIO1 | 5V |
| D24 | Wake signal | ←IN | ESP32-S3 GPIO5 (RFID wake trigger) | 3.3V* |
| USB-B | Programming | — | PC USB cable | — |

## 5.2  Mega TX2 → ESP32 Voltage Divider

Mega TX2 outputs 5V logic; ESP32-S3 GPIO44 is 3.3V tolerant. Divider: R6 = 1kΩ in series, R7 = 2kΩ to GND.

Mega TX2 ──[R6: 1kΩ]──┬──► ESP32 GPIO44 RX │ [R7: 2kΩ] │ GND Vout = 5V × 2/(1+2) = 3.33V ✓ Safe for ESP32

# 6. ESP32-S3-WROOM-1-N16R8 (U10) — Main Controller

Role: Main intelligence. Handles Wi-Fi connectivity, product database lookup, display rendering via I2C/SPI, motion sensor monitoring, and communication with SWIPE main store server (HTTP / WebSocket). 16MB Flash, 8MB PSRAM.

## 6.1  ESP32-S3 Pin Mapping

| Pin | Function | Dir | Connected To | Voltage |
| --- | --- | --- | --- | --- |
| 3V3 | Power IN | IN | 3.3V_RAIL (AMS1117 output) | 3.3V |
| GND | Ground | — | Common GND | 0V |
| EN | Enable | IN | 10kΩ pullup to 3.3V | 3.3V |
| GPIO1 | PIR OUT | ←IN | PIR Sensor HC-SR501 OUT | 3.3V |
| GPIO2 | IR Sensor 1 | ←IN | E18-D80NK S2 OUT (10kΩ pullup) | 3.3V |
| GPIO3 | IR Sensor 2 | ←IN | E18-D80NK S3 OUT (10kΩ pullup) | 3.3V |
| GPIO4 | Fuel Alert | ←IN | MAX17043 ALRT (low battery interrupt) | 3.3V |
| GPIO5 | RFID Wake | OUT→ | Arduino Mega D24 | 3.3V |
| GPIO6 | Display RST | OUT→ | Display Touch Controller RST | 3.3V |
| GPIO7 | Display INT | ←IN | Display Touch Controller INT | 3.3V |
| GPIO8 | I2C SDA | BIDIR | Display SDA + MAX17043 SDA (4.7kΩ pullup) | 3.3V |
| GPIO9 | I2C SCL | OUT→ | Display SCL + MAX17043 SCL (4.7kΩ pullup) | 3.3V |
| GPIO43 | UART0 TX | OUT→ | Mega RX2 PIN17 | 3.3V |
| GPIO44 | UART0 RX | ←IN | R6/R7 divider ← Mega TX2 PIN16 | 3.3V |
| GPIO19 | USB D− | BIDIR | USB-C connector J2 | — |
| GPIO20 | USB D+ | BIDIR | USB-C connector J2 | — |

# 7. 7" Capacitive Touch Display (DSP1)

| Signal | Display Pin | ESP32-S3 GPIO | Notes |
| --- | --- | --- | --- |
| VCC | 5V power | 5V_DISPLAY (Buck_C) | Isolated rail — prevents flicker |
| GND | Ground | Common GND | Star ground point |
| SDA (Touch) | GT911 SDA | GPIO8 (shared I2C bus) | 4.7kΩ pullup to 3.3V |
| SCL (Touch) | GT911 SCL | GPIO9 (shared I2C bus) | 4.7kΩ pullup to 3.3V |
| RST (Touch) | Reset | GPIO6 | Active LOW reset |
| INT (Touch) | Interrupt | GPIO7 | Touch event interrupt |
| SPI Data | ST7701S SPI | GPIO10–13 (MOSI/MISO/SCK/CS) | Display pixel data |

# 8. Motion & Proximity Sensors

| Ref | Model | Power | Output Pin | ESP32 GPIO | Purpose |
| --- | --- | --- | --- | --- | --- |
| S1 | HC-SR501 PIR | 3.3V | OUT → HIGH | GPIO1 | Person approach detection |
| S2 | E18-D80NK IR | 5V_LOGIC | OUT → LOW active | GPIO2 (10kΩ pu) | Item in basket — left |
| S3 | E18-D80NK IR | 5V_LOGIC | OUT → LOW active | GPIO3 (10kΩ pu) | Item in basket — right |

PIR settings: sensitivity pot to minimum, time pot to ~3 seconds, JP1 in H (re-triggerable) mode. IR sensors: detection range pot set to ~30cm.

# 9. Complete RFID Scan Signal Flow

| # | Stage | Detail |
| --- | --- | --- |
| 1 | Person Approaches Cart | PIR S1 detects body heat → GPIO1 HIGH to ESP32-S3 |
| 2 | ESP32 Wake Trigger | ESP32-S3 GPIO5 sends WAKE signal → Arduino Mega D24 |
| 3 | Item Placement Detection | IR S2/S3 detects item in basket → GPIO2/GPIO3 HIGH to ESP32-S3 |
| 4 | E710 Enable | ESP32 → Mega D24 → Mega sets D22 HIGH → Level Shifter → E710 PIN5 EN HIGH (3.3V) |
| 5 | RF Power-Up | E710 powers up RF PA — 300ms startup delay |
| 6 | Antenna Scan | E710 scans all 4 antenna ports sequentially (port hopping) |
| 7 | Tag EPC Reception | Tag EPC received → E710 sends UART frame at 115200 baud |
| 8 | UART to Arduino | E710 PIN10 TX → Level Shifter (3.3V→5V) → Mega RX1 (Serial1 PIN19) |
| 9 | EPC Parsing | Arduino Mega parses EPC frame (MultiInventory SDK protocol) |
| 10 | Bridge to ESP32 | Mega TX2 (5V) → R6/R7 voltage divider (→3.3V) → ESP32 GPIO44 RX |
| 11 | Product Lookup | ESP32-S3 receives EPC → queries product database → retrieves item info |
| 12 | Display Update | ESP32-S3 I2C (GPIO8/9) → Display SDA/SCL → item info rendered on screen |
| 13 | Scan Complete | ESP32 signals Mega → Mega sets D22 LOW → E710 EN LOW → E710 standby (≤1mA) |

⚠  E710 RF PA startup requires 300ms after EN HIGH before reliable tag reads. Add this delay in Arduino firmware before issuing inventory commands.

# 10. Decoupling Capacitors

All decoupling capacitors must be placed as close as possible to their respective IC power pins.

| Ref | Value | IC | Notes |
| --- | --- | --- | --- |
| C11 | 100μF/16V electrolytic | Arduino Mega VIN | Bulk input filter |
| C12 | 100nF ceramic | Arduino Mega VIN | Parallel with C11 |
| C13 | 100nF ceramic ×3 | Arduino Mega 5V pins | One per 5V power pin |
| C14 | 10μF/10V ceramic | ESP32-S3 3V3 pin | Bulk decoupling |
| C15 | 100nF ceramic | ESP32-S3 3V3 pin | Parallel with C14 |
| C16 | 1μF ceramic | ESP32-S3 3V3 pin | As close to module edge as possible |
| C17 | 100μF/16V electrolytic | E710 VCC | Absorbs 1600mA PA current spike |
| C18 | 10μF/16V ceramic | E710 VCC | Mid-frequency decoupling |
| C19 | 100nF ceramic | E710 VCC | HF decoupling near FPC connector |
| C20 | 100nF ceramic | Level Shifter HV (5V) | BSS138 supply bypass |
| C21 | 100nF ceramic | Level Shifter LV (3.3V) | BSS138 supply bypass |

# 11. Ground Plane & Trace Width Rules

## 11.1  Grounding Rules

- Star grounding: All GND connections meet at a single star point near the battery negative / BMS GND.

- RF ground: E710 GND and antenna cable shields connect via dedicated low-impedance trace (≥3mm wide) to star point.

- Signal ground: UART signal grounds (Mega, ESP32) share the same star point — do NOT use a separate signal ground.

- Chassis ground: Aluminium basket foil connects to star GND via a dedicated wire with ring terminal (RF cavity reference ground).

- Charging ground: TP5100 GND, BMS P−, and battery GND all converge at star point.

## 11.2  Trace Width Reference

| Trace / Net | PCB Width | Wire Gauge | Notes |
| --- | --- | --- | --- |
| 11.1V power traces | ≥ 2mm | 22 AWG | Main battery bus |
| 5V_E710 traces | ≥ 1.5mm | 24 AWG | E710 isolated rail |
| 5V_LOGIC traces | ≥ 1mm | 26 AWG | Logic supply |
| 3.3V traces | ≥ 0.8mm | 28 AWG | Low-current rail |
| UART signal traces | 0.3mm | 30 AWG | Keep as short as possible |
| RF coax (antenna) | N/A | RG316 50Ω | Dedicated — no substitution |

# 12. Bill of Materials

Estimated costs in INR as of May 2026. Suppliers: Robu.in, Amazon.in, Aliexpress, Waveshare, Mouser/Digi-Key.

| Ref | Component | Specification | Qty | Cost (INR) | Seller |
| --- | --- | --- | --- | --- | --- |
| BT1 | Li-Ion 18650 cells | Samsung 32E 3200mAh 3.7V | 6 | ₹1,080 | Quartz |
| BMS1 | 3S 10A BMS Module | 3S balance BMS, 10A, cell balancing | 1 | ₹56 | Quartz |
| U1 | Buck Converter A (E710 rail) | MP1584 12V→5V 2A | 1 | ₹40 | Quartz |
| U2 | Buck Converter B (Logic rail) | MP1584 12V→5V 1A | 1 | ₹40 | Quartz |
| U3 | Buck Converter C (Display rail) | MP1584 12V→5V 500mA | 1 | ₹40 | Quartz |
| U4 | LDO 3.3V Regulator | AMS1117-3.3 SOT-223 | 1 | ₹15 | Quartz |
| U5 | Li-Ion Charger IC | TP5100 SOP-8 (3s) | 1 | ₹55 | Quartz |
| U7 | UHF RFID Module | Impinj E710 — 8 port Reader Module | 1 | ₹10,572 | Alibaba(Ethan Lee) |
| U8 | Logic Level Shifter | BSS138 4-channel bidirectional | 1 | ₹37 | Quartz |
| U9 | Arduino Mega 2560 | ATmega2560 official board | 1 | ₹925 | Quartz |
| U10 | ESP32-S3-WROOM-1 | N16R8 (16MB Flash / 8MB PSRAM) | 1 | ₹900 | Quartz |
| DSP1 | 7" Capacitive Touch Display | SPI + I2C (GT911), ST7701S driver | 1 |  |  |
| ANT 1,2,3,4,5 | 8dBi Circular Antenna | A001 190×190mm SMA male | 5(₹1650) | ₹8250 |  |
| S1 | PIR Sensor | HC-SR501 | 1 | ₹40 |  |
| S2, S3 | IR Proximity Sensor | E18-D80NK adjustable 3–80cm | 2 | ₹40 |  |
| J1 | 12V DC Barrel Jack | 5.5×2.1mm female | 1 | ₹30 |  |
| J2 | USB-C Connector | USB 2.0 Type-C | 1 | ₹25 |  |
| L1–L3 | Inductor 47μH 3A | CDRH127/LDNP | 3 | ₹90 |  |
| C1–C9 | Electrolytic Cap 100μF/25V | Radial, input/output filter | 9 | ₹45 |  |
| C10–C21 | Ceramic Caps (100nF–10μF) | Decoupling, various packages | 12 | ₹40 |  |
| R1–R12 | Resistors 1kΩ–10kΩ | 1/4W carbon film assorted | 12 | ₹20 |  |
| D1, D2 | Indicator LEDs | 3mm red/green (charge status) | 2 | ₹10 |  |
| FOIL1 | Copper Foil Tape | Conductive adhesive, RF shielding | 1 | ₹200 |  |
| FOIL2 | Aluminium Adhesive Foil | RF cavity lining for basket | 1 | ₹200 |  |
| TOTAL ESTIMATED COST |  |  |  | ≈ ₹22,750 |  |

# 13. Critical Assembly Reminders

- NEVER connect E710 UART_RX directly to Arduino Mega TX — level shifter is mandatory or the E710 UART_RX pin will be permanently damaged.

- NEVER share 5V_E710 rail with any other load — E710 PA current spikes (1600mA burst) will brown-out other ICs.

- VERIFY all power rail voltages with a multimeter before connecting any IC.

- Aluminium foil basket lining GND must connect to the star ground point with a dedicated wire and ring terminal.

- Wire prototype in order: (1) Power rails → (2) Arduino Mega + E710 via level shifter → (3) ESP32-S3 UART bridge → (4) Display I2C → (5) Motion sensors → (6) Antenna connections last.

- Add 300ms delay in Arduino firmware after setting E710 EN HIGH before issuing UART inventory commands.

InnoCart V2  ·  SWIPE Technologies   ·  May 2026